4G T Shape HD PTZ camera

Model：AJ-CT-TW10-4G

Feature：

Single channel 1080P HD video, dual 1080P storage and transmission

Intelligent bit rate adjustment greatly reduces transmission traffic and storage space

H.265, backward compatible with H.264

4G full network communication, remote real-time monitoring, optional dual 4G

Built-in memory card, maximum 256G, combined with intelligent code rate adjustment algorithm, can greatly save storage space

Built-in WIFI, supports hotspot mode working mode

Supports remote export of videos by connecting the station to the router

Support GPS module, precise positioning

The video is superimposed with 4G signal strength information, and the signal quality is read locally and remotely in real time.

Support two-way voice intercom

Support shock-absorbing structure to improve equipment working stability

Infrared distance: 120~150 meters, optional laser light

Applications：

Suitable for Maritime Safety Administration coastal defense monitoring, port monitoring, border control monitoring, forest fire prevention, railway monitoring, airport monitoring

Specifications：

| PTZ： |  |
| --- | --- |
| Pan preset speed | 100°/S |
| Tilt preset speed | 60°/S |
| Pan manual speed control | 0°～ 60°/S |
| Tilt manual speed control | 0°～ 40°/S |
| Pan angle | 360°endless rotation |
| Tilt angle | -90°～ +90° |
| Preset position accuracy | ±0.1° |
| Preset | 256 preset positions, 8 patrol tracks |
| Wiper | Support |
| control method | RS485, built-in PELCO D/P protocol, baud rate optional from 2400Bps to 19200Bps |
| Relative humidity | <90% |
| Protection level | IP66 |
| Operating temperature | -35°C ～ +55°C |
| Power consumption | DC10.8～DC18V |
| Impact resistance requirements | ≥4G |
| Installation method | Various installation methods are optional according to the application environment |
| Weight | 7kg |

Dimension:(mm)